# MHP-001 Rev C1 - Manual Handling Procedure

Europlan Construction Work Package (CWP) – SC021 & SC038

## Purpose

Define safe manual handling requirements for installation of SC021 cleanrooms and SC038 modular rooms after materials have been delivered to the workface by others.

## Scope

50 mm rockwool sandwich panels.

4.0 m + 3.5 m stacked wall construction.

Columns, beams, ceiling members, doors, windows and flooring.

Excludes unloading and delivery to workface.

## Responsibilities

Europlan Site Manager.

Europlan Site Supervisor.

Operatives.

QA/QC Engineer.

## Key Requirements

Assess every lift.

Use mechanical assistance where manual control cannot be maintained.

Manual suction handles are handling aids only.

Never leave partially erected walls unsupported.

## Typical Crew Guide

4.0 m panel (~75 kg): 4 operatives.

3.5 m panel: 4 operatives.

5.5 m panel (~95 kg): 6 operatives if applicable.

## Hold Points

Panel damage.

Temporary Stability unavailable.

Installation sequence changes.

Drawing discrepancy.

Unsafe manual handling.

## Records

Daily POWRA.

Dynamic Risk Assessment.

Supervisor Checklist.

Inspection Records.

## Placeholder

YELLOW HIGHLIGHT: Insert SC038 room-specific handling notes when remaining drawings are issued.