| Key Guidance This section provides a quick overview of some of the key concepts in Army risk assessment. Refer to Notes section for further information. The first line of the risk assessment table, below, shows an illustrative example. Hazard is anything that may cause harm, e.g. working at height on a ladder. Risk is the chance that someone or something could be harmed by the hazard, measured by combining (multiplying) the likelihood of it happening with its impact (severity). For example, there may be a ‘possible’ likelihood that someone that is not competent could fall from a ladder (3 rating – see right) combined with a ‘moderate’ impact of multiple injuries (2 rating), which creates a score of 6 (low risk). However, the risk should be reduced to as low as reasonably practicable (ALARP) through the implementation of control measures, such as ensuring that only trained people climb the ladder. Dynamic Risk Assessment compliments generic and specific risk assessment. Regardless of completing this AF 5010, it is beholden on the person creating the risk to continue to monitor the activity and the control measures. Any changes to the activity (including the environmental conditions) or the control measures, must be addressed via the mechanism of a dynamic risk assessment such that risks remain ALARP. Note however that persons undergoing training cannot be deemed competent until their cauupability is properly assessed |  |  |  | Likelihood (L) |  | Multiplied by | Impact (I) | Equals | Risk Score Calculation. Likelihood. 1; 2; 3; 4; 5. I m p a c t; 5; 5; 10; 15; 20; 25. 4; 4; 8; 12; 16; 20. 3; 3; 6; 9; 12; 15. 2; 2; 4; 6; 8; 10. 1; 1; 2; 3; 4; 5. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 – Remote / Rare 2 – Unlikely 3 – Possible 4 – Probable 5 – Highly Probable (Almost Certain) |  |  | 1 – Minor 2 – Moderate 3 – Major 4 – Severe 5 – Critical Note: impact number is unlikely to change with control measures |  |  |  |
| 5 Step Process | Step 1 – Identify the hazards | Step 2 – Decide who might be harmed and how | Step 3 – Evaluate the risks and decide on precautions (control measures) |  | Step 4 – Record your significant findings and include in Ex / Coord instructions as necessary. Implement control measures |  |  |  |  | Step 5 – Review your risk assessment and update as necessary |

| Company: | Europlan | Assessor: | Hemant Apte |
| --- | --- | --- | --- |
| Activity (SSW) / Exercise (SST): | Cleanroom and Other Modular Room Assembly Activities CCV, MFB & VCB | Assessor’s signature: | Hemant Apte |
| Generic or Specific Risk Assessment: | Generic for Cleanroom and Other Modular Room Assembly Activities | Assessment Date: | 17/07/2026 |
| Relevant Publications / Procedures: | Vendor instructions and standard project procedures | Review Date for GRA (Step 5): | 16/08/2026 |

| (a) | (b) | (c) | (d) | (e) | (f) | (g) | (h) | (i) | (j) | (k) | (l) | (m) | (n) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Activity / element (Step 1a) | Hazards identified (Step 1b) | Who or what might be harmed and how, e.g. • personnel - fatality • staff / contractors - injury • General public - injury • Environment - spill (Step 2) | Existing control measures (Step 3a) | Assessment with existing controls |  |  | Is residual risk acceptable in the context of risk appetite for the activity? (Yes / No) – Refer to Risk Score Calculation above If Yes, move to column (n). If No, identify additional controls (Step 3e) | Reasonable additional controls that can be implemented to reduce risk to ALARP (Step 3f) | Reassessment with additional control measures |  |  | List required action(s) to instigate controls (Step 3j) |
|  |  |  |  |  | L (1 to 5) (Step 3b) | I (1 to 5) (Step 3c) | Score (L x I) (Step 3d) |  |  | L (1 to 5) (Step 3g) | I (1 to 5) (Step 3h) | Score (L x I) (Step 3i) |  |
| 1 | Access to Site | Unfamiliar with safety rules in force. | Visitors, personnel/contractors | Signage in place | 2 | 5 | 10 |  | People who have not worked on sites before are not permitted on site without a suitably experienced member of staff in attendance. Employee competency before visiting site to be determined from experience and training, e.g., EUROPLAN safety induction training or equivalent. Lone working is not allowed. Young persons (under 18 years of age) are not permitted to work on site. Unless specified by a project specific risk assessment and constant supervision and written permission from a parent or guardian. Health and safety training records to be available for review. Appropriate training to be identified before staff visit or are seconded to site. Supervisors to be competent and experienced. Information to be available to all staff and regularly reviewed for adequacy. | 1 | 5 | 5 |  |
| 2 | Inadequate lighting or visibility. | Slips, trips fall, | personnel / contractors / visitors – fractures, strains, sprains | Work during daylight hours only unless adequate plans for lighting are arranged. | 2 | 3 | 6 | Yes | Headtorches, portable task lighting for any darker areas. | 1 | 3 | 3 |  |
| 3 | Working at Height | Falls from height, Suspension trauma, fatality, dropped objects. working at height on a ladder | personnel / contractors | All operators to be trained adequately, to carry out daily safety checks and keep materials within the basket or platform. All work at height to be suitably planned and organised. | 4 | 5 | 20 | No | All equipment (harnesses, anchor points) to be checked prior to use to ensure that they are suitable for use no damage. Rescue plan to be in place and all operatives to be trained and competent for rescue. Barriers and appropriate warning signage to be erected to prevent unauthorised access into any potential drop zones. Operatives to be trained and competent in the selection and use of fall restraint/fall arrest equipment. Working at height permit required prior to start of any work at height. All work at height to be in accordance with the current working at height standards, codes of practice and statutory requirements. Tool lanyards to be used at all times. Fall arrest (e.g., harnesses) / Fall protection (e.g., safety nets, air bags, etc.) systems may not be used on site unless a risk assessment / method statement shows that:- - they can be used safely - other safer work equipment is not practicable - Adequate numbers of trained personnel are available on site to carry out the works, including emergency rescue. Fall arrest systems (e.g., harnesses) cannot be used where insufficient clearance is available below the work platform to safely arrest a fall. All working platforms, from which an employee could fall a distance liable to cause personnel injury, must be inspected every 7 days, and a written report prepared. Note – this includes ALL work platforms and not just scaffolding. While using ladders ensure the state of the ladder and inspect height is suitable to reach work point with safe use of ladder. Secure ladder firmly and ensure no other users in the area can interfere with the secured ladder. Barricade or man guard the ladder work area. Follow HSE directive on use of the ladders. | 1 | 5 | 5 |  |
|  |  |  |  |  |  |  |  |  | EUROPLAN staff shall not operate any Mobile Elevating Work Platforms (MEWP) Before using check on stability arrangements, suitable position, protection of outriggers, max wind operating speed, ground conditions, loading capacities, and certification of MEWP. MEWP’s must be operated by trained personnel only. Ensure there is safe access / egress, platform edge protection before using and that you have knowledge of emergency procedures for evacuation. Use of fall arrest equipment attached to a suitable anchorage point in the basket or platform. Check for presence and avoid overhead cables or other obstructions. Erect warning signs and demarcation if potential conflict. |  |  |  |  |
| 4 | Electricity & Testing | Electricity / Electrocution | personnel / contractors / visitors / equipment / environment – death, burns, blinding, | Only trained personnel to work | 4 | 5 | 20 | No | LOTO provisions to be adhered to. Treat all cabling as live until proved dead. Barrier off test areas. No live work unless authorised. Follow the correct. Any cables seen on site shall be treated as source of energy and live. Obtain written consent of the party who are responsible for that cable prior to working in vicinity of the cables. | 1 | 5 | 6 |  |
| 5 | Manual Handling | Manual Handling – incorrect carrying or lifting of large or heavy loads | Operatives, staff, visitors - minor injuries Muscular sprains, strains, fractures, lacerations, crushing etc. through incorrect lifting techniques | Use correct lifting techniques and operatives to be trained adequately. | 3 | 3 | 6 | Yes | All operatives to be trained in manual handing, where possible all lifts to be by mechanical means. Where not possible kinetic lifting methods to be used. Assess the best way to lift before moving the load. Ensure a clear route with no obstructions and remove potential hazards before lifting. Team lift loads over the maximum person weights Manual handling risk assessments completed 2. Personnel complete annual manual handling training. | 1 | 3 | 3 |  |
| 6 | Mechanical Lifting Operations | Lifting- Overloading of lifting appliance, overloading or incorrect use of lifting gear, incorrect positioning of lifting appliance, insecure attachment of load, improper methods of use of equipment, failure of equipment due to lack of maintenance, incorrect signals. | Operatives, contractors | All personnel to be suitably trained. | 5 | 5 | 25 | No | Staff to make themselves aware of site operations immediately on entering the site, and of the dangers from being struck by lifting equipment and / or lifted objects. All lifting requires a lifting permit to be in place prior to the start of works. PPE to be worn; particularly safety helmets, safety glasses, boots, and high visibility clothing. DO NOT enter lifting / danger zones during lifting operations. Obey any ‘exclusion zones’ / safety signage relating to lifting operations. Obey the instructions of banksmen, and other designated personnel, whilst on site. Do not give signals, or directions, to crane drivers. DO NOT enter the slew area of mobile cranes, and other lifting appliances. DO NOT position yourself underneath any load being lifted. Use cross bar, if required, on Pipe Lifters | 1 | 5 | 5 |  |
| 7 | Noise | Loss of Hearing- Tinnitus, fatigue and tiredness | Operatives, visitors, contractors Tinnitus, fatigue, tiredness | Noise monitored | 3 | 3 | 9 | No | Where personnel will be required to work in situations where levels of noise are likely to be encountered, the Project Manager will ensure that full information is obtained before work commences on the levels and frequencies of noise. - First Action Level: Daily personal noise exposure (LEP,d) > 80dB(A). Hearing protectors to be provided. - Second Action Level: Daily personal noise exposure (LEP,d) > 85dB(A). noise levels to be reduced below 85dB, or where this is not possible adequate hearing protection to be provided, worn, and signage erected “Hearing Protection Must be Worn” All persons working on construction sites should have hearing protection available to them. Supervisory staff to be made aware of need to wear hearing PPE in any area designated by the Contractor as an ear protection zone. Staff to be made aware of need to avoid watching noisy operations for long periods, e.g., piling, compressors, pneumatic tools, saws, pumps, and mixers. Also ‘peak’ noises, e.g., riveting, cartridges. As a rule of thumb if you have to raise your voice to be heard, 2m or less from the person, you need to wear hearing protection. Always wear hearing protection in areas where there are sign “Hearing Protection Must be Worn” Ensure that you don’t expose your fellow workers to your noise. Move noise sources away from the working areas, where possible. If possible, shield noisy processes, work behind a wall or some other sound absorbing material. NOTE:- make sure that the ear protection you wear is worn correctly, if it is not, it may be ineffective. Ensure that ear plugs are a good fit and correctly inserted. Use disposable earplugs once only. Clean your hands before touching all types of earplugs. Earmuffs should fit the head all around the seal of the earmuffs. Earmuffs must be worn the correct way around, i.e., Left side to left ear, Right side to right ear. Ensure that muff seals are in a good condition. If you have difficulties with wearing the ear protection provided report it to your supervisor. | 1 | 3 | 3 |  |
| 8 | Emergency Procedures | Lack of Preparedness - Fire on the construction site or in accommodation. Release of chemicals. Collapse of structure / stored materials. Utility strike, explosion, or high-pressure fluid release Intrusion into working space by errant vehicles Destruction of Traffic Management by vehicles. | Operatives, Staff, visitors – Death, Severe injury, burns, broken bones | Emergency procedures noted in induction video. | 3 | 5 | 15 | No | Site Rules and Emergency Procedures to be detailed by the Main / Lead Contractor, in the Safety & Health Plan. Emergency contacts to be readily available at work location(s) Local fire points with appropriate extinguishers, to secure safe exit from danger, to be provided and used in accordance with emergency procedure instructions only. In the event of an emergency - Alert other personnel in the area, and isolate danger areas (if it is safe to do so) In the event of a fire, and if there is no danger by doing so, try to put out the fire with the extinguishers provided. Remember this equipment will only be effective on a small fire - you must catch it before it gets hold. Leave the building / site by the nearest designated escape route Walk, do not run. Do not stop to collect personal belongings. Inform Site Management Go to the Assembly Point When at the assembly point report to site supervision. Do not leave the Assembly Point until instructed to do so In the event of a road traffic accident (at roadworks) immediately calls the emergency services. Suspend works on site and remove personnel from areas of immediate danger (if it is safe to do so). If it is safe to do so close off hazardous areas of the carriageway; until complete traffic management can be restored. Assist the emergency services when they arrive on the scene | 1 | 5 | 5 |  |
| ?? | General work | Wall Panel Installation– incorrect carrying or lifting of large or heavy panels | Operatives, staff, visitors - minor injuries Muscular sprains, strains, fractures, lacerations, crushing, pinched fingers etc. through incorrect lifting techniques and uncoordinated slide fits. | Use correct lifting techniques and operatives to be trained adequately. | 3 | 3 | 9 | Yes | Manual handling risk assessments completed 2. Personnel complete annual manual handling training. 3. Prepare topple arrest arrangements and engage upon panel placement on the base frame. | 1 | 3 | 3 | Confirm provision of topple arrest arrangements at the top of the panel or on either one side before commencing the manual lift in position over base frame. Ensure firm engagement of vacuum lifting handles if and when used. Check for any disengagements due to protective film before lifting. If protective film is observed tampered, remove protection film and engage the handles on to the bare panel surface. |
| 7 | Hot Works – Drilling, cutting and grinding | Flying debris, hot debris, cut risks, harmful dust, excessive noise | Operatives, staff, visitors | Wear all relevant site PPE including eye protection and gloves. | 5 | 5 | 25 | Yes | All hot works require a hot work permit to be issued prior to start of work. Area will be barriered off, fire extinguisher to be at cutting area. Fire watch to continue for 1 hour after hot work operations stop. All operatives to be suitably trained and comptent. Area to have barriers installed around. Face shield to be used when cutting or grinding. Before commencing any work. Inspect area and specific locations for presence of any other services alive or dead which may impact from grinding, drilling, cutting actions. | 1 | 5 | 5 |  |
| 8 | Use of work equipment hand tools – power tools. | Cuts, entanglement with tool bits, eye damage, penetrating wounds | Operative, contractors | Wear standard and task specific PPE. | 2 | 2 | 4 | Yes | Pre-use visual check of plant and equipment before use, defects reported and tools in quarantine if unsuitable. Regular inspections to be undertaken on all tools. Only trained and experienced operatives to use power tools. Hearing protection to be worn when using power tools. Ensure all certification available for lifting devices 6 monthly. Use the correct tool for the job. | 1 | 2 | 2 |  |
| 8 | Vibrating Tools and Equipment | Industrial Disease, Disability, Reduction in Dexterity | Operatives, contractors |  | 4 | 4 | 16 |  | All operatives exposed to hand-arm vibration trained in (HAV) awareness. All vibrating tools and equipment used should be of low vibration specification where practicable. Hand-arm vibration to be calculated to determine restrictions of tool use throughout the day. | 1 | 4 | 4 |  |
| 9 | Multiple trades working in one area. | Injuries, noise, | Operatives, staff | Work in areas that are available | 3 | 3 | 9 | Yes | Organise works to be done in a specific order and talk to other trades to avoid any works | 1 | 3 | 3 |  |
| 10 | Heavy Plant | Crushing, broken bones, noise, | Operatives, staff, visitors | Wear standard PPE. | 4 | 3 | 12 | Yes | Stick to walkways, wear hi-viz PPE, signal machine worker before passing, | 2 | 3 | 6 |  |
| 11 | Forklift | Moving materials | Operatives, staff, visitors | All operators to be trained adequately and to carry out daily safety checks and adjust machinery for each lift. | 3 | 5 | 15 | Yes | Use banksman for loading and unloading to reduce the risk of damage or falling objects. | 1 | 5 | 5 |  |

| Authorising Officer | Name | Job title | Date | Signature |
| --- | --- | --- | --- | --- |
| Existing and additional controls agreed | Hemant Apte | Interface Manager | 19/05/2026 | Hemant Apte |
| Where risk is elevated up the HoE to confirm additional controls implemented |  |  |  |  |

# NOTES

| Risk = Likelihood x Impact Likelihood; Definition. 5; Highly Probable (Almost Certain); Is expected to occur in most circumstances. 4; Probable; Will probably occur at some time, or in most circumstances. 3; Possible; Fairly likely to occur at some time, or some circumstances. 2; Unlikely; Is unlikely to occur, but could occur at sometime. 1; Remote / Rare; May only occur in exceptional circumstances. Impact; Definition (Health Safety and Environment). 5; Critical; - Multiple fatalities or permanent, life changing injuries. - Permanent loss or damage beyond remediation of an important and publicly high-profile natural resource, area or species. - Multiple incidents causing a major environmental impact.. 4; Severe; - A single death or multiple life-threatening injuries. - Severe damage over a wide area and/or on a prolonged basis to a natural resource, including controlled waters, or geography requiring multi-year remediation. - Single incident causing a major environmental effect or multiple incidents causing significant effect.. 3; Major; - Single life changing injury or multiple injuries which have a short-term impact on normal way of or quality of life. - Moderate damage to an extended area and/or area with moderate environmental sensitivity (scarce/ valuable) requiring months of remediation. - Single incident causing significant environmental impact.. 2; Moderate; - Multiple injuries requiring first aid. - Moderate damage to an area, and that can be remedied internally. - Multiple incidents causing minor environmental effect.. 1; Minor; - An Injury requiring first aid - Limited short-term damage to an area of low environmental significance/ sensitivity - Incidents causing minor environmental impacts. | Step 5 - Review the generic risk assessment and update if necessary - All generic risk assessments should be regularly reviewed at a frequency proportional to the risk prior to any controls being proposed. In practice generic risk assessments should be reviewed at least annually, or more frequently: • where required by local instructions/procedures. • if the safe execution of the activity relies on stringent supervision and/or adherence to a safe system of work. • if there is reason to doubt the effectiveness of the assessment. • following an accident or near miss. • following significant changes to the task, process, procedure, equipment, personnel or management. • following the introduction of more vulnerable personnel (e.g. persons under 18 or pregnant persons). Risk Management. Risk Rating; Authorisation; How Risk should be managed. 1 – 3 (Very Low); Supervisors; Review periodically to ensure conditions have not changed and working within ALARP and risk appetite.. 4 – 9 (Low); Supervisors. 10 – 12 (Medium); Managers; Good risk mitigations to ensure that the impact remains ALARP and tolerable. Re-assess frequently to ensure conditions remain the same.. 15 – 16 (Medium to High); Managers; Requires active management – review of desired outcome with additional resources or change to output requirements.. 20 (High); Directors; Contingency plans may suffice together with limited risk mitigations to achieve risk ALARP and tolerable.. 25 (Very High); Directors; Work capability where the required outcome impacts on work capability.. |
| --- | --- |